===============================================================================

TITLE  : LapLink cable specifications.

DATE   : 18-08-1994

AUTHOR : C. Holz.

EMAIL  : cholz@wilbur.mbark.swin.oz.au

===============================================================================

Serial link cable.

------------------

Make a serial cable with two female DB25 (or female RS232) connectors.

Seven connections are required.  Wire the cable as follows:

9 pin       25 pin          25 pin      9 pin

=====       ======          ======      =====

pin 2       pin 3   <--->   pin 2       pin 3  (Receive-Transmit)

pin 3       pin 2   <--->   pin 3       pin 2  (Transmit-Receive)

pin 4       pin 20  <--->   pin 6       pin 6  (DTR---DSR)

pin 5       pin 7   <--->   pin 7       pin 5  (Grd---Grd)

pin 6       pin 6   <--->   pin 20      pin 4  (DSR---DTR)

pin 7       pin 4   <--->   pin 5       pin 8  (RTS---CTS)

pin 8       pin 5   <--->   pin 4       pin 7  (CTS---RTS)

===============================================================================

Parallel link cable.

--------------------

Make a parallel wiring box with male DB-25 connectors at both ends.

Fifteen connections are required.

25 pin          25 pin

======          ======

pin 01  <---->  pin 01

pin 02  <    >  pin 15

pin 03  <    >  pin 13

pin 04  <    >  pin 12

pin 05  <    >  pin 10

pin 06  <    >  pin 11

pin 10  <    >  pin 05

pin 11  <    >  pin 06

pin 12  <    >  pin 04

pin 13  <    >  pin 03

pin 14  <---->  pin 14

pin 15  <    >  pin 02

pin 16  <---->  pin 16

pin 17  <---->  pin 17

pin 25  <---->  pin 25  (Grd---Grd)

===============================================================================

Null modem plugs.

-----------------

The best way I have seen to make you own laplink cables is to purchase two DB25

wiring boxes.  One Male to Male, the other Female to Female.

The Male to Male is wired up as a Parallel null modem, and the Female to Female

as the Serial null modem.

All that is needed then is a standard serial EXTENSION cable (Male to Female

DB25).

By plugging the Serial Null modem into the Male end, you have a full serial

laplink cable with Female to Female connectors.

Or visa-versa by plugging the Parallel Null modem into the Female end, you have

a full parallel laplink cable with Male to Male connectors.

The only single problem with this is, that the serial null modem will not plug

into a standard RS232 9 pin plug.  This can be overcome by purchasing one or

two convertors (9 pin Female to 25 pin male).

===============================================================================

Laplink extension cable.

------------------------

Instead of forking out large amounts of money for an extension cable for your

current cable or for the null modem, and extension cable can be made from the

following specs.

Make a cable with one female and one male DB-25 connector at either end.

Fifteen wires are required.  Four jumper wires are required between pins.

25 pin          25 pin

======          ======

pin 01          pin 01

pin 02          pin 02

pin 03          pin 03

pin 04          pin 04

pin 05          pin 05

pin 06          pin 06

pin 07 / 25     pin 07 / 25

pin 10          pin 10

pin 11 / 20     pin 11 / 20

pin 12          pin 12

pin 13          pin 13

pin 14          pin 14

pin 15          pin 15

pin 16          pin 16

pin 17          pin 17

===============================================================================

Information.

------------

These specifications are complete to my knowledge.  The specs allow in laplink

what is known as Accelerated Transfer, which is much faster than Standard

transfer.

These cable have been fully tested for DOOM.  To play DOOM over parallel link,

it is necessary to acquire a file called DOOMPAR.ZIP produced by an author

called Andrew who produced a serial driver for use with DOOM r1.1.

One warning with using the parallel connection with DOOM, that some computers

do not work and a connection is never found.

===============================================================================